# Impact of the WOCL on Short-Call Reserve Flightcrews under FAR 117 versus FAR 121 subpart Q

By

Joshua Foltz, First Officer, Allegiant Air

Garret Healy, Software Engineer, Jeppesen Inc.

Table of Contents

Introduction:	4

Abstract:	4

Definitions:	5

Airport/standby reserve (ASB):	5

Duty:	5

Duty Break:	5

Flight Duty Period (FDP):	5

Flightcrew member (FCM):	5

Flight Time:	5

Long-call reserve (LCR):	6

Pre-Flight Duties:	6

Reserve availability period (RAP):	6

Reserve flightcrew member:	6

Short-call reserve (SCR):	6

Split duty:	6

Suitable accommodation:	6

Window of circadian low (WOCL):	6

Assumptions:	7

Brief time:	7

Debrief time:	7

Minimum Connect Time:	7

Permissible Extensions to FDP:	7

Rest Requirements:	7

Minimum Duty Break:	7

Travel Time:	7

Call out:	7

Flight Segments:	7

Acclimated:	8

Nature of reserve assignments:	8

Specific regulatory provisions applied:	8

Daily Flight Time Limitations (FTL):	8

Daily FDP Limitations:	9

Consecutive nighttime operations.	10

Cumulative Limitations.	10

Reserve status.	10

Objective comparisons of combined RAP FDP:	11

FAR 121 subpart Q	11

FAR 117	13

Conclusions:	15

Recommendations:	15

Interactive spreadsheets:	15

Reference Information:	16

# Introduction:

In the course of the preparatory research and development, this material has been developed to assist the readers in the understanding of FAR part 117.

The documentation is provided “AS IS” and is solely intended to provide a general understanding of the author’s interpretation of the new FAA mandated FAR Regulations.

The author makes no representations and disclaims any and all responsibility for the completeness or accuracy of the documentation. The authors’ reserve the right, at their discretion, to change or modify the documentation as deemed appropriate.

Copyright © 2012-2013, Understanding FAR Part 117 – A Generic Interpretation. All rights reserved.

# Abstract:

The author’s developed the following tables as a crew planning tool. These tables demonstrate how strict FAR Part 117 work rules are in relation to allowed duty time when compared to FAR part 121 subpart Q.

The particular problems surrounding WOCL rules associated with reserve assignments, pose vastly different situations under FAR 117 dependent upon the type of reserve associated with the FDP assigned, and the time of day as to when the reserve begins and the time of day that the FDP begins, while under FAR 121 subpart Q, no distinction in accordance to the diurnal variance had ever been stipulated.

Our models will differ from our previous articles, in that we will concentrate less upon duty period construction, and more upon flight crewmember Rostering / Lines construction, as the constraints now posed under FAR 117 are dependent upon consecutive days associated with the WOCL, as well as the cumulative FDP constraints under FAR 117 and the cumulative flight time constraints under FAR 121 subpart Q.

Airlines must schedule efficient Flight Duty Periods and carefully plan when Flight Duty Periods

begin as well as how many segments are involved.

In order to schedule efficient pairings the Airlines should consider looking at productive hours (flight time) versus non-productive hours (FDP time)/Combined RAP/ FDP (Duty time).

# Definitions:

## Airport/standby reserve (ASB):

Is defined in FAR part 117.

means a defined duty period during which a flightcrew member is required by a certificate holder to be at an airport for a possible assignment.

## Duty:

Is defined in FAR part 117.

Means any task that a flightcrew member performs as required by the certificate holder, including but not limited to flight duty period, flight duty, pre- and post-flight duties, administrative work, training, deadhead transportation, aircraft positioning on the ground, aircraft loading, and aircraft servicing.

## Duty Break:

Must meet the following criteria:

- Is scheduled before the start of the FDP.

- Is at least 3 hours.

- Is not provided until after the first segment In the FDP has been completed.

- Is provided between 22:00 and 05:00 local time.

- Is measured from the time that the flightcrew member reaches the suitable accommodation.

- Duty breaks cannot be reducible from scheduled. Even during delays, the scheduled duty  break cannot be reduced

## Flight Duty Period (FDP):

Is defined in FAR part 117.

Means a period that begins when a flightcrew member is required to report for duty with the intention of conducting a flight, a series of flights, or positioning or ferrying flights, and ends when the aircraft is parked after the last flight and there is no intention for further aircraft movement by the same flightcrew member. A flight duty period includes the duties performed by the flightcrew member on behalf of the certificate holder that occur before a flight segment or between flight segments without a required intervening rest period. Examples of tasks that are part of the flight duty period include deadhead transportation, training conducted in an aircraft or flight simulator, and airport/standby reserve, if the above tasks occur before a flight segment or between flight segments without an intervening required rest period.

## Flightcrew member (FCM):

Is a pilot, flight engineer, or flight navigator assigned to duty in an aircraft during flight time.

Is defined in FAR part 1.

## Flight Time:

Is defined in FAR part 1.

Commences when an aircraft moves under its own power for the purpose of flight and ends when the aircraft comes to rest after landing.

## Long-call reserve (LCR):

Is defined in FAR part 117.

means that, prior to beginning the rest period required by 117.25, the flightcrew member is notified by the certificate holder to report for a flight duty period following the completion of the rest period.

## Pre-Flight Duties:

Includes reporting for an assignment, acknowledging flights to be performed while on duty, will be operated in accordance to FAA regulations, including understanding of the expected conditions that will affect the flight(s) to be performed. Inspections of the aircraft for flight readiness, etc.

## Reserve availability period (RAP):

Is defined in FAR part 117.

means a duty period during which a certificate holder requires a flightcrew member on short call reserve to be available to receive an assignment for a flight duty period.

## Reserve flightcrew member:

Is defined in FAR part 117.

means a flightcrew member who a certificate holder requires to be available to receive an assignment for duty.

## Short-call reserve (SCR):

Is defined in FAR part 117.

means a period of time in which a flightcrew member is assigned to a reserve availability period.

## Split duty:

Is defined in FAR part 117.

means a flight duty period that has a scheduled break in duty that is less than a required rest period.

## Suitable accommodation:

Is defined in FAR part 117.

means a temperature-controlled facility with sound mitigation and the ability to control light that provides a flightcrew member with the ability to sleep either in a bed, bunk or in a chair that allows for flat or near flat sleeping position. Suitable accommodation only applies to ground facilities and does not apply to aircraft on-board rest facilities.

## Window of circadian low (WOCL):

Is defined in FAR part 117.

means a period of maximum sleepiness that occurs between 0200 and 0559 during a physiological night.

Note: The WOCL is based off of flight crew members last acclimated time.

# Assumptions:

## Brief time:

The amount of pre-flight duty before the flight departs the gate. Typically ranges between 0:45 to 1:00; generally contained within a CBA; for the purposes of this paper we will use 1:00.

## Debrief time:

The amount of post-flight duty after the flight arrives at the gate. Typically ranges between 0:15 to 0:30; generally contained within a CBA; for the purposes of this paper we will use 0:15.

## Minimum Connect Time:

The minimum time from when an aircraft arrives at the gate, to the time when the aircraft moves away from the gate to allow for sufficient to unload/load passengers and baggage, and make any required service needs for the aircraft before departure as well as performance of pre-flight checklists, for the purposes of this paper we will use 0:30.

## Permissible Extensions to FDP:

Under FAR 117.19, it is permissible to operate beyond the scheduled limitations in FAR 117.13 by 0:30, extensions up 2:00 may be conducted before departure under certain conditions, for the purposes of this paper we will use 0:00.

## Rest Requirements:

For the purposes of this paper, a FCM will always be in compliance with the following requirements:

- Under 117.25, FCM’s are required to have been scheduled for and have been given a rest period of no less than 30 hours in the 168 consecutive hours preceding the start of a FDP.

- Under 121 subpart Q, FCM’s are required to have been scheduled for and have been given a rest period of no less than 24 hours in the 7 calendar days preceding the scheduled completion of any flight subject to this subpart.

## Minimum Duty Break:

FAR 117.15 mandates that the minimum time spent in a suitable accommodation is 3:00.

## Travel Time:

FAR 117.15, mandates that the time to travel to/from the location used as a suitable accommodation is not part of the required duty beak, generally, the location is relatively close-by; for the purposes of this paper we will use 0:15 each way for travel time, this will equate to 0:30.

## Call out:

Airlines have various policies regarding minimum time from when a FCM is notified to report for an FDP and the start time of the FDP. For the purposes of the paper, it will be assumed that the FCM is notified at the start time of a RAP, and the FDP assigned will start following a 2:00 call-out.

## Flight Segments:

FAR 117.13 contains different FDP limitations based upon the number of flight segments, for the purpose of this paper all FDP will have 3 flight segments.

## Acclimated:

FAR 117.13, mandates that a 0:30 penalties be applied to the FDP limitations in Table B when a FCM reports for an FDP in a un-acclimated state. For purposes of this paper a FCM will be assumed to be in an acclimated state, no penalties shall be applied.

# Nature of reserve assignments:

Long Call Reserve (LCR) – a FCM is notified that an FDP or SCR assignment has been made and the span of time from point of notification to the start of the assignment will allow for a rest period and any required travel time.

Airport/Standby Reserve (ASB) – a FCM is at the airport and is subject to immediate assignment to an assignment, which includes Flight Time.

Short Call Reserve (SCR) –a FCM is expected to report for the FDP without an intervening rest period. Since the FCM may need to travel to the location where the FDP will start, most airlines will have a minimum amount of time to allow for the ‘call out’.

The amount of time from the start of the short-call reserve until the report for the FDP is known as RAP, Reserve Availability Period, it includes the ‘call out time’, and may not exceed 14 hours.

# Specific regulatory provisions applied:

## Daily Flight Time Limitations (FTL):

- Part 117 has Daily FTL’s governed under 117.11.

  - Based upon FDP start time, the limitation is either 8:00 or 9:00 as defined in Table A.

  - Evaluated on a leg by leg basis, FCM may not continue a flight if before takeoff it is known that the FTL will be violated.

Table A

- Part 121 has Daily FTL’s governed under 121.471(a)(4).

  - The scheduled limitation is 8:00.

  - A FCM may continue the assigned FDP as long as the flight itinerary is the same as what the FCM began with no further constraints (legal to start, legal to finish), else a reevaluation at the point of rescheduling must take place.

## Daily FDP Limitations:

- Part 117 has Daily FTL’s governed under 117.13.

  - Based upon FDP start time, the scheduled limitation is defined in Table B.

  - Extensions to the assigned FDP are permitted in accordance with 117.19.

Table B

- Part 121 does not actually have FDP limitations, instead a complicated system based upon 121.471(b) and (c):

  - A scheduled flight time in the 24 hours preceding the scheduled completion of a flight segment.

  - A required rest period in the 24 hours preceding the actual expected completion of a flight segment. The may be reduced due to operational contingencies before takeoff.

  - A compensatory rest period will be required following the release of a duty period that is preceded by a reduced rest period, this will effectively limit the duty period to a maximum of 16 or 15 hours dependent upon the scheduled flight time in the 24 hour period, and the compensatory rest period must commence within 24 hours of the start of the reduced rest period.

| Scheduled Flight time in 24 Hours | Required Rest Period | Permitted Reduced Rest Period | Compensatory Rest Period |
| --- | --- | --- | --- |
| < 8 | 9 | 8 | 10 |
| >= 8 and < 9 | 10 | 8 | 11 |
| >= 9 | 11 | 9 | 12 |

FAR 121.471 (b) and (c)

## Consecutive nighttime operations.

- Part 117 has Consecutive nighttime operations governed under 117.27.

  - Up to 5 consecutive calendar days where a FDP infringes upon any portion of the WOCL, will be permitted under the following conditions:

    - Each FDP will have a rest opportunity of at least 2 hours,

    - The rest opportunity measured from the time that the flightcrew member reaches the suitable accommodation,

    - The rest oppurtunity must comply with the conditions specified in  117.15(a), (c), (d), and (e).

  - Otherwise, the limitation is 3 Consecutive nighttime operations.

- Part 121 subpart Q has no limitations.

Authors note: An early morning FDP assignment can also be considered a night duty when the FDP starts at any time between 02:00 and 05:59 (inclusive).

## Cumulative Limitations.

- Part 117.23 has the following Cumulative Limitations:

  - Cumulative flight time limitations of:

    - 100 hours in any 672 consecutive hours.

    - 1000 hours in any 365 calendar days

  - Cumulative FDP limitations of:

    - 60 hours in any 168 consecutive hours.

    - 190 hours in any 672 consecutive hours

- Part 121 subpart Q has the following Cumulative Limitations:

  - Cumulative flight time limitations of:

    - 30 hours in any 7 calendar days.

    - 100 hours in any calendar month.

    - 1000 hours in any calendar year

## Reserve status.

- Part 117 has specific limitations for FCM assigned to reserve assignments governed   under 117.21.

  - All reserve assignments are considered to be duty for the purposes of determination of all rest requirements under 117.25.

  - Airport/Standby (ASB) Reserve is considered FDP and is subject to the FDP Limitations (117.13) in Table B and the Flight Time Limitations (117.11) in Table A using the start time of the assignment. ASB is subject to the Consecutive nighttime operations under 117.27 as well as the rest requirements under 117.25.

  - Short-Call Reserve (SCR) is not considered FDP, but is considered to be on RAP. RAP has specific limitations for while on RAP and combined RAP and FDP limitations for FDP’s assigned during the RAP. RAP’s are subject to the rest requirements under 117.25.

  - Long-Call Reserve (LCR) is not considered FDP and is not subject to the rest requirements under 117.25, but before being assigned to and FDP or SCR assignment the rest requirements must be satisfied, and should the FDP start before the WOCL and continue into the WOCL at least 12 hours notification must have been given.

- While Part 121 subpart Q has no specific limitations for FCM assigned to reserve assignments, A FCM is considered to be on duty, and when assigned to a duty that contains flight time, will still be subject to the pertinent flight time limitations and rest requirements under section 121.471(b) and (c).

# Objective comparisons of combined RAP FDP:

| Brief | Debrief | Turn Time | Travel To/From | Duty Break | Min Call Out |
| --- | --- | --- | --- | --- | --- |
| 1:00 | 0:15 | 0:30 | 0:15 | 2:00 | 2:00 |

### FAR 121 subpart Q

|  |  | 3 Flight Segments |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FDP Start |  | Max FDP | Max RAP + FDP | Eff Flight Time | Split FDP |
| 0:00 | 3:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 4:00 | 4:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 5:00 | 5:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 6:00 | 6:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 7:00 | 11:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 12:00 | 12:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 13:00 | 16:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 17:00 | 19:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 20:00 | 21:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 22:00 | 22:59 | 14:45 | 15:00 | 8:00 | 14:45 |
| 23:00 | 23:59 | 14:45 | 15:00 | 8:00 | 14:45 |

FDP Efficiency = Daily Flight Time / Daily FDP

RAP FDP Efficiency = Daily Flight Time /(Daily FDP + (FDP Start - RAP Start))

| 4 Days - FAR 121 subpart Q |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Day 1 | Day 2 | Day 3 | Day 4 | Cum. Values |  |
| RAP start |  |  |  |  |  |
| 2:00 | 2:00 | 2:00 | 2:00 |  |  |
| FDP start |  |  |  |  |  |
| 4:00 | 4:00 | 4:00 | 4:00 |  |  |
| Required FDP end |  |  |  |  |  |
| 17:00 | 17:00 | 17:00 | 17:00 |  |  |
| Earliest start of Next RAP |  |  |  |  |  |
| 2:00 | 2:00 | 2:00 | 2:00 |  |  |
| Daily Flight Time |  |  |  | FT 7 days |  |
| 7:30 | 7:30 | 7:30 | 7:30 | 30:00 |  |
| Daily FDP |  |  |  | FDP 168 |  |
| 13:00 | 13:00 | 13:00 | 13:00 | 52:00 |  |
| FDP Efficiency |  |  |  | Avg Eff |  |
| 57.69 | 57.69 | 57.69 | 57.69 | 57.69 |  |
| RAP FDP Efficiency |  |  |  | Avg Eff |  |
| 50.00 | 50.00 | 50.00 | 50.00 | 50.00 |  |

| 5 Days - FAR 121 subpart Q |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Day 1 | Day 2 | Day 3 | Day 4 | Day 5 | Cum. Values |
| RAP start |  |  |  |  |  |
| 2:00 | 2:00 | 2:00 | 2:00 | 2:00 |  |
| FDP start |  |  |  |  |  |
| 4:00 | 4:00 | 4:00 | 4:00 | 4:00 |  |
| Required FDP end |  |  |  |  |  |
| 17:00 | 17:00 | 17:00 | 17:00 | 17:00 |  |
| Earliest start of Next RAP |  |  |  |  |  |
| 2:00 | 2:00 | 2:00 | 2:00 | 2:00 |  |
| Daily Flight Time |  |  |  |  | FT 7 days |
| 6:00 | 6:00 | 6:00 | 6:00 | 6:00 | 30:00 |
| Daily FDP |  |  |  |  | FDP 168 |
| 13:00 | 13:00 | 13:00 | 13:00 | 13:00 | 65:00 |
| FDP Efficiency |  |  |  |  | Avg Eff |
| 46.15 | 46.15 | 46.15 | 46.15 | 46.15 | 46.15 |
| RAP FDP Efficiency |  |  |  |  | Avg Eff |
| 40.00 | 40.00 | 40.00 | 40.00 | 40.00 | 40.00 |

### FAR 117

|  |  | 3 Flight Segments |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FDP Start |  | Max FDP | Max RAP + FDP | Eff Flight Time | Split FDP |
| 0:00 | 3:59 | 9:00 | 13:00 | 7:30 | 12:00 |
| 4:00 | 4:59 | 10:00 | 14:00 | 8:00 | 13:00 |
| 5:00 | 5:59 | 12:00 | 16:00 | 9:00 | 14:00 |
| 6:00 | 6:59 | 12:00 | 16:00 | 9:00 | 14:00 |
| 7:00 | 11:59 | 13:00 | 16:00 | 9:00 | 14:00 |
| 12:00 | 12:59 | 13:00 | 16:00 | 9:00 | 14:00 |
| 13:00 | 16:59 | 12:00 | 16:00 | 9:00 | 14:00 |
| 17:00 | 19:59 | 11:00 | 15:00 | 9:00 | 14:00 |
| 20:00 | 21:59 | 11:00 | 15:00 | 8:00 | 14:00 |
| 22:00 | 22:59 | 10:00 | 14:00 | 8:00 | 13:00 |
| 23:00 | 23:59 | 10:00 | 14:00 | 8:00 | 13:00 |

FDP Efficiency = Daily Flight Time / Daily FDP

RAP FDP Efficiency = Daily Flight Time /(Daily FDP + (FDP Start - RAP Start))

#### Early Starts

| Early Starts |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Day 1 | Day 2 | Day 3 | Day 4 | Day 5 | Cum. Values |
| RAP start |  |  |  |  |  |
| 2:00 | 2:00 | 2:00 | 2:00 | 2:00 |  |
| FDP start |  |  |  |  |  |
| 4:00 | 4:00 | 4:00 | 6:00 | 4:00 |  |
| Required FDP end |  |  |  |  |  |
| 14:00 | 14:00 | 14:00 | 16:00 | 14:00 |  |
| Earliest start of Next RAP |  |  |  |  |  |
| 0:00 | 0:00 | 0:00 | 2:00 | 0:00 |  |
| Daily Flight Time |  |  |  |  | FT 7 days |
| 7:45 | 7:45 | 7:45 | 7:45 | 7:45 | 38:45 |
| Daily FDP |  |  |  |  | FDP 168 |
| 10:00 | 10:00 | 10:00 | 10:00 | 10:00 | 50:00 |
| FDP Efficiency |  |  |  |  | Avg Eff |
| 77.50 | 77.50 | 77.50 | 77.50 | 77.50 | 77.50 |
| RAP FDP Efficiency |  |  |  |  | Avg Eff |
| 64.58 | 64.58 | 64.58 | 64.58 | 64.58 | 64.58 |

#### Late Finishes

| Late Finishes (Using Duty Break) |  |  |  |  |  |  | Late Finishes (No Duty Break) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Day 1 | Day 2 | Day 3 | Day 4 | Day 5 | Cum. Values |  | Day 1 | Day 2 | Day 3 | Day 4 | Day 5 | Cum. Values |
| RAP start |  |  |  |  |  |  | RAP start |  |  |  |  |  |
| 13:00 | 13:00 | 13:00 | 13:00 | 13:00 |  |  | 13:00 | 13:00 | 13:00 | 13:00 | 13:00 |  |
| FDP start |  |  |  |  |  |  | FDP start |  |  |  |  |  |
| 15:00 | 15:00 | 15:00 | 15:00 | 15:00 |  |  | 15:00 | 15:00 | 15:00 | 15:00 | 15:00 |  |
| Required FDP end |  |  |  |  |  |  | Required FDP end |  |  |  |  |  |
| 5:00 | 5:00 | 5:00 | 5:00 | 5:00 |  |  | 3:00 | 3:00 | 3:00 | 1:59 | 3:00 |  |
| Earliest start of Next RAP |  |  |  |  |  |  | Earliest start of Next RAP |  |  |  |  |  |
| 15:00 | 15:00 | 15:00 | 15:00 | 15:00 |  |  | 13:00 | 13:00 | 13:00 | 11:59 | 13:00 |  |
| Daily Flight Time |  |  |  |  | FT 7 days |  | Daily Flight Time |  |  |  |  | FT 7 days |
| 8:45 | 8:45 | 8:45 | 8:45 | 8:45 | 43:45 |  | 9:00 | 9:00 | 9:00 | 8:44 | 9:00 | 44:44 |
| Daily FDP |  |  |  |  | FDP 168 |  | Daily FDP |  |  |  |  | FDP 168 |
| 11:00 | 11:00 | 11:00 | 11:00 | 11:00 | 55:00 |  | 12:00 | 12:00 | 12:00 | 10:59 | 12:00 | 58:59 |
| FDP Efficiency |  |  |  |  | Avg Eff |  | FDP Efficiency |  |  |  |  | Avg Eff |
| 79.55 | 79.55 | 79.55 | 79.55 | 79.55 | 79.55 |  | 75.00 | 75.00 | 75.00 | 79.51 | 75.00 | 75.84 |
| RAP FDP Efficiency |  |  |  |  | Avg Eff |  | RAP FDP Efficiency |  |  |  |  | Avg Eff |
| 67.31 | 67.31 | 67.31 | 67.31 | 67.31 | 67.31 |  | 64.29 | 64.29 | 64.29 | 67.27 | 64.29 | 64.88 |
| 4 th Consecutive Night |  |  | 5 th Consecutive Night |  |  |  |  |  |  |  |  |  |

#### Night Duties

| Night Duties (Using Duty Break) |  |  |  |  |  |  | Night Duties (No Duty Break) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Day 1 | Day 2 | Day 3 | Day 4 | Day 5 | Cum. Values |  | Day 1 | Day 2 | Day 3 | Day 4 | Day 5 | Cum. Values |
| RAP start |  |  |  |  |  |  | RAP start |  |  |  |  |  |
| 19:00 | 19:00 | 19:00 | 19:00 | 19:00 |  |  | 19:00 | 19:00 | 19:00 | 19:00 | 19:00 |  |
| FDP start |  |  |  |  |  |  | FDP start |  |  |  |  |  |
| 21:00 | 21:00 | 21:00 | 21:00 | 21:00 |  |  | 21:00 | 21:00 | 21:00 | 21:00 | 21:00 |  |
| Required FDP end |  |  |  |  |  |  | Required FDP end |  |  |  |  |  |
| 10:00 | 10:00 | 10:00 | 10:00 | 10:00 |  |  | 8:00 | 8:00 | 8:00 | 1:59 | 8:00 |  |
| Earliest start of Next RAP |  |  |  |  |  |  | Earliest start of Next RAP |  |  |  |  |  |
| 20:00 | 20:00 | 20:00 | 20:00 | 20:00 |  |  | 18:00 | 18:00 | 18:00 | 11:59 | 18:00 |  |
| Daily Flight Time |  |  |  |  | FT 7 days |  | Daily Flight Time |  |  |  |  | FT 7 days |
| 7:45 | 7:45 | 7:45 | 7:45 | 7:45 | 38:45 |  | 8:00 | 8:00 | 8:00 | 2:44 | 8:00 | 34:44 |
| Daily FDP |  |  |  |  | FDP 168 |  | Daily FDP |  |  |  |  | FDP 168 |
| 10:00 | 10:00 | 10:00 | 10:00 | 10:00 | 50:00 |  | 11:00 | 11:00 | 11:00 | 4:59 | 11:00 | 48:59 |
| FDP Efficiency |  |  |  |  | Avg Eff |  | FDP Efficiency |  |  |  |  | Avg Eff |
| 77.50 | 77.50 | 77.50 | 77.50 | 77.50 | 77.50 |  | 72.73 | 72.73 | 72.73 | 54.85 | 72.73 | 70.91 |
| RAP FDP Efficiency |  |  |  |  | Avg Eff |  | RAP FDP Efficiency |  |  |  |  | Avg Eff |
| 64.58 | 64.58 | 64.58 | 64.58 | 64.58 | 64.58 |  | 61.54 | 61.54 | 61.54 | 39.14 | 61.54 | 57.06 |
| 4 th Consecutive Night |  |  | 5 th Consecutive Night |  |  |  |  |  |  |  |  |  |

# Conclusions:

FAR 121 subpart Q, is less efficient when compared to FAR 117, this is due to the weekly flight time limitation of 30 hours in 7 calendar days, this limits the daily flight time to an average of 6:00 across 5 calendar days, and 7:30 hours across 4 calendar days.

FAR 121 subpart Q, is not constrained by diurnal variations, thereby, the average FDP efficiency and the average Combined RAP FDP efficiency remains constant across all time frames at 46.15/40.00 for 5 calendar days, and 57.19/50.00 for 4 calendar days.

FAR 117, showed the worst efficiencies for Night Duties, that did not apply a Duty Break of 2:00, day 4, was severely constrained to a very short FDP/Combined RAP FDP with a very low flight time. The average efficiency was 70.91/57.06 for FDP/Combined RAP FDP, respectfully. When Night Duties apply the Duty Break, the efficiencies improve to 77.50/64.58.

FAR 117, showed the same type of trend with Late Finishes, with no Duty Break demonstrating lower efficiencies with 75.48/64.88 for FDP/Combined RAP FDP, respectfully, again, day 4, showed the lowest performance due the constrained FDP/Flight time. When Late Finishes Duties apply the Duty Break, the efficiencies improve to 79.55/67.31. Late Finishes Duties had the highest efficiencies due to high daily FDP/Flight Time values associated with diurnal variations.

FAR 117, showed Early Start Duties had average efficiencies of 77.50/64.58 for FDP/Combined RAP FDP, respectfully. Day 4, was constrained to have a FDP start no earlier than 6:00 due to the inability to apply a Duty Break.

# Recommendations:

The use of duty breaks associated with split duties for both Night Duties and Late finishes improves efficiencies.

Early starts, pose the most compelling problem when RAPs are assigned for more than 3 consecutive days, it is understood that not every day a FCM will actually be assigned an FDP that will infringe upon the RAP, but the potential does exist.

# Interactive spreadsheets:

# Reference Information: